PIC 16F877 - CALM instructions

| Code 14 bits | Microchip | CALM |  | Flags | # Operation |
| --- | --- | --- | --- | --- | --- |
| 110000 kkkkkkkk | movlw val | Move | #Val,W | [none] |  |
| 000000 1fffffff | movwf reg | Move | W,Reg | [none] |  |
| 001000 0fffffff | movf reg,0 | Move | Reg,W | [Z] |  |
| 001000 1fffffff | movf reg,1 | Test | Reg | [Z] |  |
| 000000 01100010 | option | Move | ## W,Option | [none] |  |
| 000000 01100101 | tris porta | Move | W,TrisA 1 | [none] |  |
| 000000 01100110 | tris portb | Move | W,TrisB 1 | [none] |  |
| 000000 01100111 | tris portc | Move | W,TrisC 1 | [none] |  |
| 111110 kkkkkkkk | addlw val | Add | #Val,W | [C,D,Z] | #Val+W -> W |
| 000111 0fffffff | addwf reg,0 | Add | Reg,W | [C,D,Z] | Reg+W -> W |
| 000111 1fffffff | addwf reg,1 | Add | W,Reg | [C,D,Z] |  |
| 111100 kkkkkkkk | sublw val | Sub | W,#Val,W | [C2,D,Z] | #Val+(-W) -> W |
| 000010 0fffffff | subwf reg,0 | Sub | W,Reg,W | [C2,D,Z] | Reg+(-W) -> W |
| 000010 1fffffff | subwf reg,1 | Sub | W,Reg | [C2,D,Z] | Reg+(-W) -> Reg |
| 111001 kkkkkkkk | andlw val | And | #Val,W | [Z] |  |
| 000101 0fffffff | andwf reg,0 | And | Reg,W | [Z] |  |
| 000101 1fffffff | andwf reg,1 | And | W,Reg | [Z] |  |
| 111000 kkkkkkkk | iorlw val | Or | #Val,W | [Z] |  |
| 000100 0fffffff | iorwf reg,0 | Or | Reg,W | [Z] |  |
| 000100 1fffffff | iorwf reg,1 | Or | W,Reg | [Z] |  |
| 111010 kkkkkkkk | xorlw val | Xor | #Val,W | [Z] |  |
| 000110 0fffffff | xorwf reg,0 | Xor | Reg,W | [Z] |  |
| 000110 1fffffff | xorwf reg,1 | Xor | W,Reg | [Z] |  |
| 001110 0fffffff | swapf reg,0 | Swap | Reg,W | [none] | Reg  swapped Reg |
| 001110 1fffffff | swapf reg,1 | Swap | Reg | [none] | Reg  Reg sw. Reg  W |
| 000001 1fffffff | clrf reg | Clr | Reg | [Z=1] |  |
| 000001 00000000 | clrw | Clr | W | [none] |  |
| 000000 01100100 | clrwdt | ClrWDT |  | [none] |  |
| 000000 01100011 | sleep | Sleep |  | [none] |  |
| 001001 1fffffff | comf reg,1 | Not | Reg | [Z] | Reg  .not.Reg |
| 001001 0fffffff | comf reg,0 | Not | Reg,W | [Z] | Reg  Reg .not.Reg  W |
| 001010 1fffffff | incf reg,1 | Inc | Reg | [Z] | Reg+1  Reg |
| 001010 0fffffff | incf reg,0 | Inc | Reg,W | [Z] | Reg  Reg Reg+1 W |
| 000011 1fffffff | decf reg,1 | Dec | Reg | [Z] | Reg-1  Reg |
| 000011 0fffffff | decf reg,0 | Dec | Reg,W | [Z] | Reg  Reg Reg-1 W |
| 001101 1fffffff | rlf reg,1 | RLC | Reg | [C] | Reg  .rlc.Reg |
| 001101 0fffffff | rlf reg,0 | RLC | Reg,W | [C] | Reg  Reg .rlc.Reg  W |
| 001100 1fffffff | rrf reg,1 | RRC | Reg | [C] | Reg  .rrc.Reg |
| 001100 0fffffff | rrf reg,0 | RRC | Reg,W | [C] | Reg  Reg .rrc.Reg  W |
| 0100bb bfffffff | bcf reg,b | Clr | Reg:#b | [none] |  |
| 0101bb bfffffff | bsf reg,b | Set | Reg:#b | [none] |  |
| 001111 1fffffff | incfsz reg,1 | IncSkip,EQ | Reg | [none] | Reg+1  Reg |
| 001111 0fffffff | incfsz reg,0 | IncSkip,EQ | Reg,W | [none] | Reg  Reg Reg+1 W |
| 001011 1fffffff | decfsz reg,1 | DecSkip,EQ | Reg | [none] | Reg-1  Reg |
| 001011 0fffffff | decfsz reg,0 | DecSkip,EQ | Reg,W | [none] | Reg  Reg Reg-1 W |
| 0110bb bfffffff | btfsc reg,b | TestSkip,BC | Reg:#b | [none] |  |
| 0111bb bfffffff | btfss reg,d | TestSkip,BS | Reg:#b | [none] |  |
| 101kkk kkkkkkkk | goto addr | Jump | Addr | [none] |  |
| 100kkk kkkkkkkk | call addr | Call | Addr | [none] |  |
| 110100 kkkkkkkk | retlw val | RetMove | #Val,W | [none] |  |
| 000000 00001000 | return | Ret |  | [none] |  |
| 000000 00001001 | retfie | RetI |  | [int] |  |
| 000000 00000000 | nop | Nop |  | [none] |  |

| bcf status,0; ## CLRC; Clr F:#0. bsf status,0; SETC; Set F:#0. btfss status,0; Skip,CS; TestSkip,BS F:#0. btfsc status,0; Skip,CC; TestSkip,BC F:#0. btfss status,2; Skip,EQ Skip,ZS; TestSkip,BS F:#2. btfsc status,2; Skip,NE Skip,ZC; btfsc status,2. |  |
| --- | --- |
| Note 1: Move W,TrisD, Move W,TrisE does not exist. Select in Bank1 Note 2: Processor add the 2-s complement; Carry value is inverted compared to most other μP |  |
| All instructions execute in 4 clock cycles (1 μS at 4 MHz) except the Skip, Jump, Call and Ret that takes the double Compare for equality: Xor W,Reg or Xor Reg,W Compare for Inequality: Sub instruction, but be careful Except I/O, 16F87x, and other 14-bit instruction PICs have the same instruction set. jdn060828 |  |