| CH-1092 Belmont, Switzerland Tel +41 21 728-6156, Fax 728-6157 Email info@didel.com, www.didel.com |  |
| --- | --- |

Filehttp://www.didel.com/slow/mip/Mdxx.doc

# Md4x – 4/6-channels

# Md2x – 2/3-channels

# Miniature controllers for base radio or IR receiver

Didel controllers have a PPM input provided by a radio or IR receiver. Outputs are transistor bridges of 0.6 Amps. Several models exist to match your model constraints. They are based on two PC boards with Bahoma magnets.  They are compatible with the Emir and Radir transmitter, and apparently also with the Tanaka transmitters.

| Md4x with 4 bidirectional bridge and bahoma plug 0.58g with magnets and connectors 60 CHF, ca 39 Euros or 48 USD | Md2x with two bidirectionnal bridges 0.38g with magnets and connectors 50 CHF, ca 33 Euros or 40 USD |
| --- | --- |

Software options:

| Md40 Helicos, blimp see Md4x doc | Md41 3 or 4-channel planes Md4x doc | Md42 Twin motor planes Md4x doc |
| --- | --- | --- |
|  |  | MIB42S4 – not yet programmed |
| Md20 see Md20 doc For Gasparin or rubber motors and gliders | Md21 see Md21 doc Simple 2-channel plane | Md21s2 see Ms21s2 doc 3 channel plane with 2 servos |

Radios transmit the joystick positions as PPM signals, supposed to be min 1ms, max 2ms, center 1.5ms. Return-spring joysticks return a pulse close to 1.5ms. Trottle is supposed to be in non active position, with a value close to 1ms or 2ms pulse, depending on the radio. At power-up, the controllers checks that the PPM information exists and is in the good range

Default channel sequence on MDxx controller

|  | chan1 | chan2 | chan3 | chan4 | Application | Availability |
| --- | --- | --- | --- | --- | --- | --- |
| 2ms= | no powerright | right | up | right up |  |  |
| Md20 | not decoded | Rudder | Elevator | ignored | Gasparin, rubber, glider | now |
| Md21 | Throttle | Rudder | ignored | ignored | Simple plane | now |
| Md22 | Mot/act | Mot/act | ignored | ignored | Blimp |  |
| Md22M | MotLeft | MotRight | ignored | ignored | Twin with diff. drive |  |
| Md21S2 | Throttle | Rudder servo | Elevator servo | ignored | Normal plane with servo | now |
| Md2S2M | Throttle | Elev1 | Elev2 | ignored | V-tail plane |  |
| Md4x |  |  |  |  |  |  |
| Md40 | Mot1 | Mot2 | Mot3 | Mot4 | Blimp, helico | now |
| Md41 | Throttle | Ailerons | Elevator | Rudder | Normal aerobatic | now |
| Md41M | Throttle | Elev1 | Elev2 | Rudder | V-tail |  |
| Md42 | Throttle1 | Throttle2 | Elevator | Rudder | Twin motor | now |
| Md41S2 | Throttle | Aileron/Rudder | Elevator | 2 servos | e.g. with landing gear |  |
| Md41S4 | Throttle | Aileron servo | Elevator servo | Rudder servo | chan 5 landing gear |  |

Click on first column items to get details

Note on PPM 1-2ms to actuator conversion

On several radios, the PPM signal never goes below 1.05 ms, or above 1.95 ms. MDxx controllers provide a 0% to 100% action in this range. This limit slightly the sensitivity, but guarantee a 100 % action with all radios, even if trimmed. Indeed, in the figure below, the straight line is a stair, with 32 steps for the full range.

0411226jdn/041009us