# Smile-NG – Programming the different PICs   270401

# Dllpic4.dll Script.ini - Configuration words for PICs

| Type | CP_Off_Mask dans Dllpic4.dll | Config word (PWRTE WDT off) |
| --- | --- | --- |
| 16C84 | 16’3FF0 = 16368 | RC 3FFB=16379 (16’1B = 27 sur Smaky) XT 3FF9=16377 (16’19 = 25 sur Smaky) |
| 16F84A 16F84A | 16’3FF0 = 16368 | RC 3FF3=16371 (16’13 = 19 sur Smaky) XT 3FF1=16377 (16’11 = 17 sur Smaky) |
| 16F87x | 16’3F30 = 16176 | RC 3F3B=16185 XT 3F39=16185 (Boden=0) 3F71=16241 (Boden=1) HS 3F3A |
| 12C508/9 | 16’FE8 = 4072 | 6 I/O FEA=4074 |
| 12C505 | 16’FD0 = 4048 | 12I/O FD4= 4052 |
| 16F67x | 16’3F60 = 16224 | 16I/O 3F64 = 16228 |
| 16F62x | 16’3F00 = 16128 | 6 I/O 3F18 = 16152 |
| 16C710/11 | 16’3FB0 = 16304 | RC 3FB3 = 16307 |
| 16C54 | 16’FF8 = 4088 | RC FFB = 4091 |

# Script.ini – Assignments after  Convert 1 Download

| 16F84 RC PicG84 | PIC84_4 OSC=3 CP=0 WDT=0 PWT=0 verify=0 special=0 cfg=241 type=3 str=PIC16F84 |
| --- | --- |
| 16F84 XT | PIC84_4 OSC=1 CP=0 WDT=0 PWT=0 verify=0 special=0 cfg=16377 type=3 str=PIC16F84 |
| 16F87x XT PicG870 Boden=0 | PIC84_4 OSC=1 CP=0 WDT=0 PWT=0 verify=0 special=0 cfg=16185 type=3 str=PIC16F870 |
| 12C508-6I/O 12C509-6I/O | PIC84_4 OSC=4 CP=0 WDT=0 PWT=0 verify=0 special=0 cfg=4074 type=0 str=PIC12C508 |
| 12C505-12I/O | PIC84_4 OSC=4 CP=0 WDT=0 PWT=0 verify=0 special=0 cfg=4074 type=0 str=PIC12C505 |
| 16F670-16I/O 16F671-16I/O | PIC84_4 OSC=4 CP=0 WDT=0 PWT=0 verify=0 special=0 cfg=4074 type=0 str=PIC12C671 |
| 16F627-6I/O 16F628-6I/O | PIC84_4 OSC=4 CP=0 WDT=0 PWT=0 verify=0 special=0 cfg=4074 type=0 str=PIC12C627 |
| 16C710 RC 16C711 RC | PIC84_4 OSC=4 CP=0 WDT=0 PWT=0 verify=0 special=0 cfg=16307 type=0 str=PIC12C710 |
| 16C54 RC | PIC84_4 OSC=4 CP=0 WDT=0 PWT=0 verify=0 special=0 cfg=4091 type=0 str=PIC12C54 |

SmileNG has been developed by S. Gerlach as a student at EPFL

The PIC programmer software has been developed by Michel Pahud when he was at LAMI-EPFL